European Innovation Council (EIC) & European Innovation Council and SMEs Executive Agency Pathfinder Open

GA: 101130161

Mapping the brain-spinal cord interaction towards understanding and treatment of movement disorders

Move2Treat

https://move2treat.org/

WP3

FEP unstructured fibre waveguide inscription

Technical Coordinator	Christos Markos

Organisation		DTU Electro

Start date of project		01-Jul-2024

Move2Treat Ref::		Move2Treat _CUT_250210_A_WP3

Author(s)			Andreas Ioannou

| Dissemination level |  |  |
| --- | --- | --- |
| PU | Public | X |
| SEN | Sensitive |  |

This project has received funding from the European Union’s Horizon EIC Pathfinder programme under grant agreement No. 101130161. Views and opinions expressed are however those of the author(s) only and do not necessarily reflect those of the European Union or European Innovation Council and SMEs Executive Agency (EISMEA). Neither the European Union nor the granting authority can be held responsible for them

1  2	 3   4	5		  6	  7	   8 	    9	    10

a)

b)

Fig. 1 Microscope image of Waveguide fabrication in unstructured FEP fibre a) at the beginning, b) at the end of a 5000 um long waveguide

- Attempts 1,2,4,5,9 – Failed due to damage – high energy.

Table 1 Inscription parameters on the waveguide fabrication.

|  | - Energy / Repetition rate | - Inscription speed | - Comments |
| --- | --- | --- | --- |
| - 3 | - 25nJ @ 1 KHz | - 400 um/s | - Better positioned waveguide in the centre of fibre … less damage. Better index modification. |
| - 6 | - 27nJ @ 1 KHz | - 400 um/s | - Good inscription, similar index modification as 10…. Less burning good candidate |
| - 7 | - 37nJ @ 1 KHz | - 500 um/s | - Too much damage. |
| - 8 | - 27nJ @ 1 KHz | - 500 um/s | - Good inscription, sightly weak index modification. |
| - 10 | - 28nJ @ 1 KHz | - 300 um/s | - Good inscription, slightly stronger index modification than 8. Some burned spots probably due to fiber inhomogeneity |

Fig. 2 Red light lunched into the FEP obvious scattering from the modified area

Fig. 3 FBG in the centre not placed in any waveguide